B300 GPU P2P Bandwidth – Brief Report  (Ricardo Jacomin – Juan Perafan)

This report summarizes three peer-to-peer (P2P) GPU communication benchmarks implemented to evaluate different layers of GPU data-transfer performance. Although all three involve GPU-to-GPU transfers, they measure distinct behaviors and should not be directly compared.

## 1. benchmark_peer_to_peer_orig — Hardware Link Measurement

Purpose:
Measures raw GPU-to-GPU link performance between a single pair of GPUs (0 ↔ 1), using direct send/receive operations.

Characteristics:

- Only two GPUs involved

- No topology optimization

- Pure link-level bandwidth

What it measures:

- Bidirectional bandwidth:

BW=2×bytestime

Expected result:

- Lower values for small messages (latency bound)

- Plateau at large sizes

- Peak around ~1300 GB/s, matching NVLink expectations

Interpretation:
Validates the underlying hardware interconnect.

## 2. benchmark_peer_to_peer_streamed — Application-Level Multi-GPU Behavior

Purpose:
Evaluates how efficiently a Python-level implementation can drive full-mesh communication across all GPUs.

Characteristics:

- Every GPU sends to every other GPU

- Loops and streams allow limited overlap

- Impacted by Python scheduling overhead

What it measures:

- Injected bandwidth per GPU:

BW=bytestime

Expected result:

- Bandwidth lower than hardware peak

- Typically tens to hundreds of GB/s

Interpretation:
Represents more realistic application-level communication behavior rather than hardware capability.

## 3. benchmark_peer_to_peer_nccl — NCCL-Style Collective Communication

Purpose:
Simulates a multi-GPU communication pattern similar to NCCL collectives, achieving better overlap and scheduling.

Characteristics:

- All-to-all communication

- Closer to optimized NCCL algorithms

- Implicit handling of peer interactions

What it measures:

- Injected bandwidth

- Bus bandwidth:

BWbus=BWinj×(world size-1)

Expected result:

- Latency-limited at small sizes

- Plateau at large sizes

- Typical bus BW: 1250–1650 GB/s

Interpretation:
A realistic approximation of optimized collective performance.

Results:

| Test Version P2P | Size | BW 1-dir (GB/s) | BW Bi-dir (GB/s) | Injected BW (GB/s) | Bus BW (GB/s) | Notes / Interpretation |
| --- | --- | --- | --- | --- | --- | --- |
| Original (01) | 10 MB | ~173 | 346 | NA | NA | Small message the latency dominates, link not saturated |
|  | 100 MB | ~274 | 548 | NA | NA | Approaching link efficiency |
|  | 500 MB | ~632 | 1265 | NA | NA | ✔ Near NVLink peak (bidirectional link test) |
| Streamed P2P (4 GPUs) | 10 MB | NA | NA | 28.6 | NA | Heavy overhead, no saturation |
|  | 100 MB | NA | NA | 42.6 | NA | Partial overlap, still inefficient |
|  | 500 MB | NA | NA | 50.3 | NA | ❌ Very low the scheduling/serialization dominates |
| NCCL-style (collective) | 10 MB | NA | NA | ~196 | 588 | Startup/latency regime |
|  | 100 MB | NA | NA | ~374 | 1122 | Near saturation |
|  | 500 MB | NA | NA | ~414 | 1241 | ✔ Steady-state NVLink bandwidth |

1. Summary

| P2P | Scope | Metric | What It Measures | Comparable to NCCL? |
| --- | --- | --- | --- | --- |
| original | 2 GPUs | Bidirectional BW | Hardware link | ❌ No |
| streamed | N GPUs | Injected BW | Application-level behavior | ❌ No |
| nccl-style | N GPUs | Bus BW | Collective communication | ✔ Yes |

2. Key Findings

2.1 OriginalP2P (GPU 0 ↔ 1)

• Measures raw NVLink point-to-point link bandwidth. 
• At 500 MB message size, bidirectional bandwidth reaches ~1265 GB/s, indicating healthy NVLink link performance. 
• Does not represent multi-GPU or collective performance—only single-link validation.

2.2 Streamed P2P (Full mesh across 4 GPUs)

• Measures application-level communication using Python and asynchronous send/receive patterns. 
• Injected bandwidth ranges only 28–50 GB/s (very low). 
• Bottlenecks are software-related: Python overhead, imperfect overlap, kernel launch serialization, and contention. 
• Not a hardware limitation—NVLink is not the cause of low throughput.

2.3 NCCL-style Collective Benchmark

• Measures optimized collective communication using NCCL-like algorithms (ring/tree). 
• Achieves ~414 GB/s injected bandwidth and ~1241 GB/s bus bandwidth at 500 MB. 
• Matches expected NVLink performance for collective operations (1250–1650 GB/s range). 
• This is the *only* test that is directly comparable to vendor-reported NVLink performance.

3. Interpretation

The three methods evaluate different layers of the GPU communication stack and are therefore not directly comparable.

• Original P2P validates hardware link bandwidth. 
• Streamed P2P captures software-layer efficiency (and its limitations). 
• NCCL-style shows optimal, topology-aware multi-GPU communication and aligns with expected NVLink bus bandwidth.

4. Final Conclusion

Only the NCCL-style benchmark represents real-world optimized performance and should be used when comparing to expected NVLink bandwidth numbers. Original P2P confirms hardware health, while Streamed P2P reflects application-level overhead rather than hardware capability.

The following results summarize the stability and performance of the Original peer-to-peer (P2P) transfer benchmark

| Peer-to-Peer Transfer | P2P Transfer 10 MB (bidirectional) | 341.98 GB/s, 0.061 ms |
| --- | --- | --- |
| Peer-to-Peer Transfer | P2P Transfer 1024 MB (bidirectional) | 1302.21 GB/s, 1.649 ms |
| Peer-to-Peer Transfer | P2P Transfer 2048 MB (bidirectional) | 1320.94 GB/s, 3.251 ms |

  The results show stable high-bandwidth performance for large message sizes (1–2 GB).

  The peak bandwidth is reached at ~1024 MB, with only a minor increase at 2048 MB, indicating that the link is operating near its maximum throughput.

  Latency grows predictably with message size, consistent with expected transfer times for direct GPU-to-GPU communication.